### UNIT-II

AC CIRCUITS AND TRANSFORMERS

- What is meant by average value?

The average value of an alternating current is defined as the D.C. current which transfers across any circuit the same charge as is transferred by that alternating current during the same time.

Average value = Area under the curve for one complete cycle

Base (time period)

### Define RMS value.

The steady current which when flowing through a given resistor for a given time produces the same amount of heat as produced by the alternating current when flowing through the same resistor for the same time is called R.M.S or effective value of the alternating current.

RMS value=  area the square for one cycle

Period

### Define form factor.

It is defined as the ratio of RMS value of current (voltage) to the average value of current (voltage).

Form factor = RMS value

Average value

### Define peak factor.

It is defined as the ratio of maximum value of current (voltage) to the RMS value of current (voltage).

Peak factor = Maximum value

RMS value

### Distinguish between power and energy.

| Sl.No | Power | Energy |
| --- | --- | --- |
| 1. | The rate at which work is done is called electric power. | The capacity to do work or the total work done is called energy. |
| 2. | Power (P) = VI Where V—Voltage I – Current | Energy = VIt Where V—Voltage I --- Current t --- Time |
| 3. | Unit : Watts | Unit : Watt-Hour |

- What is meant by electric power? Give different forms of expression for electric power.

The rate at which work is done in an electric circuit is called electric power. Electric Power = Work done in electric circuit

Time Electric Power (P) = VI watts

= I2R watts

= V2/R watts

### Draw power triangle.

- Define time period and frequency of an alternating quantity.

Time period:

The time taken by an alternating quantity to complete its one cycle is known as time period denoted by T and measured in Seconds (s). After every T seconds, the cycle of an alternating quantity repeats.

### Frequency:

The number of cycles completed by an alternating quantity per second is known as frequency. It is denoted by ‘f’ and it is measured in cycles/second or hertz (Hz). Frequency is reciprocal of time period.

f =1/T

### Explain the terms (a) instantaneous value (b) wave form and (c) cycle for an ac signal.

Instantaneous Value:

The value of an alternating quantity at a particular instant is known as its instantaneous value.

### Waveform:

The graph of instantaneous values of an alternating quantity plotted against time is called as waveform.

### Cycle:

Each repetition of a set of positive and negative instantaneous values of an alternating quantity is called as cycle.

### Define power factor.

The cosine of angle between voltage and current is known as power factor. It is also defined as the ratio of resistance to the impedance.

Power factor = cos φ = R/Z

# Define a transformer.

The transformer is a static piece of apparatus by means of which electrical power is transformed from one alternating current circuit to another with desired change in voltage and current without any change in the frequency. It works on the principle of mutual induction.

### Distinguish between core and shell type transformer.

In core type, the windings surround the core considerably and in shell type the core surround the winding.

### What is an ideal transformer and how does it differ from a practical transformer.

An ideal transformer is one which does not involve any power loss and does not have any change in frequency. It decreases or increases the input voltage.

Practical transformer is one which involves power loss and does not have any change in frequency. It decreases or increases the input voltage.

# Write down the emf equation of a transformer.

Emf induced in primary coil E1 = 4.44 fΦmN1 volt Emf induced in secondary coil E2 = 4.44fΦmN2 volt

Where f is the frequency of AC input Φm is the maximum value of flux in the core N1, N2 are the number of primary and secondary turns.

# Why transformers are rated in kVA?

Copper loss of a transformer depends on current and iron loss on voltage. Hence total losses depend on Volt- Ampere and not on the power factor. That is why the rating of transformers is in kVA and not in kW.

# Why the core of a transformer is laminated?

The purpose of laminating the co re in a transformer is to reduce eddy current loss.

# Define voltage transformation ratio of transformer.

The ratio of secondary induced emf to primary induced emf is called as voltage regulation ratio denoted by K

E2/E1 = N2/N1 = K

# What are the losses occurring in a transformer?

  - Core losses

  - Copper losses

# What is meant by core or iron losses?

Core or iron losses are caused as the core gets subjected to an alternating flux.

# What is meant by copper loss?

The copper losses are due to the power wasted in the form of I2R due to the resistances of the primary and secondary windings.

# Define all day efficiency.

All day efficiency is the ratio energy (in kwh) delivered in a 24 hours period to the energy (in kwh) input for the same length of time.

All day efficiency = Output in kwh/Input in kwh (for 24 hrs)

PART-C

- Distinguish between 1) apparent power 2) active power and 3) reactive power in AC circuits.

- Derive the expression for RMS value for a sinusoidal waveform.

## A series circuit has R=10 Ω, L=50mH, and C=100μF and is supplied with 200V,

50Hz. Find (i) Impedance (ii) current (iii) power (iv) power factor (v) voltage drop across the each element.

- Draw the construction diagram of single phase transformer. Also explain the working principle of the same.

- What are tests required to draw the equivalent circuit of a single phase transformer? How are they conducted?

- With neat circuit diagrams, explain the open-circuit and short-circuit tests conducted on single- phase transformer. How to obtain regulation and efficiency from these tests?