UNIT IV ANALOG COMMUNICATION

- Define communication.

The term communication refers to the process of transfer of data/ message from one place to other.

# What are the basic types of communication?

  - Wire or line communication.

  - Wireless or radio communication.

# What are the types of signals?

    - Analog sinal 2.Digital signal

# Differentiate analog and digital signal.

Analog signal is one in which the signal intensity varies in a smooth fashion over time. There is no break or discontinuities in the signal.

Digital signal is one in which the signal intensity maintains a constant level for some period of time and then changes to another constant level.

# What is meant by modulation?

The message signal is super imposed on a high frequency carrier signal before transmission. This process is referred to as modulation. Modulation is done with the help of a device known as modulator.

# What is meant by demodulation?

The process of recovering original message signal from the modulated carrier is known as demodulation. The device which performs demodulation is called as demodulator.

# Define modulation rate

It is defined as the rate at which signal level is changed depending on the nature of the format used to represent the digital data. It is measured in Bauds or symbols per second.

# What are the types of modulation?

    - Amplitude modulation

    - Angle modulation

a. Frequency modulation b. Phase modulation.

# What is the need for modulation?

    - To reduce height of antenna.

    - To avoid mixing of signals.(interference)

    - To increase the range of communication.

    - To allow multiplexing of signals.

    - To allow adjustments in bandwidth.

# Define modes of propagation. What are the modes of propagation in optical fiber?

Modes are the various parts the light can take in traveling down the fiber.

Single mode: There is only one path for light to take down the cable, it is called single mode.

Multi mode : If there are more than one path for the light to talk down the cable, it is called multi mode.

  - Define depth of modulation.

It is defined as the ratio between message amplitude to that of carrier amplitude.

# Define Amplitude modulation

In amplitude modulation, the amplitude of the carrier is varied in accordance with the instantaneous amplitude of the message signal. i.e. whenever the message signal reaches maximum amplitude, the amplitude of the carrier is also made higher and whenever the message signal reaches minimum amplitude, the amplitude of the carrier is also minimum.

# Define Frequency modulation

In frequency modulation, the frequency of the carrier is varied in accordance with the instantaneous amplitude of the message signal. i.e. whenever the message signal reaches maximum amplitude, the frequency of the carrier is also made higher and whenever the message signal reaches minimum amplitude, the frequency of the carrier is also minimum.

# Define Phase modulation.

In phase modulation, the phase of the carrier is varied in accordance with the instantaneous amplitude of the message signal.

# What is meant by modulation index for FM wave?

Modulation index for FM is defined as the ratio of maximum frequency deviation to modulating signal frequency. The value of modulation index for FM is normally greater than 1.

# What is bandwidth for AM wave?

Bandwidth is the difference between highest and lowest frequencies in the transmission of AM wave.

Bandwidth = Upper side band frequency – Lower side band frequency

= (fc+fm)-(fc-fm) = 2fm

Bandwidth of AM wave is equal to twice the modulating signal frequency.

# What is meant by modulation index for AM wave?

Modulation index for AM is defined as the ratio of modulating signal amplitude to carrier signal amplitude. The value of modulation index for AM lies between 0 and 1.

ma = Vm / Vc

  - What is the degree of modulation?

-Under modulation, m<1

-Critical Modulation, m=1

-Over modulation, m>1

  - State the Carson’s rule.

An approximate rule for the transmission bandwidth of an FM signal generated by a single tone modulating signal of frequency.

# Why frequency modulation is known as constant bandwidth system?

The bandwidth for FM wave is given as

Bandwidth = 2(∆f + fm)

# The maximum value of frequency deviation is fixed at 75 KHz for commercial broadcasting.

If fm = 50 Hz then Bandwidth = 150.1 KHz If fm = 500 Hz then Bandwidth = 151 KHz If fm = 5 KHz then Bandwidth = 160 KHz

Thus, although the modulation frequency changes from 50 Hz to 5 KHz , the bandwidth occupied by the spectrum alters very little, from 150.1 KHz 160 KHz. So frequency modulation is known as constant bandwidth system.

# PART C

    - Explain about communication systems.

    - Write elaborately about modulation principles and its techniques.

    - Write down the comparison between Amplitude modulation and Frequency modulation.

    - What are the types of Signals and explain Analog and Digital Signals

    - Explain Modulation and Demodulation

    - Explain about Amplitude modulation.

    - Explain about frequency modulation.

    - Explain about pulse modulation.