UNIT V COMMUNICATION SYSTEMS

- Define amplitude Modulation.

Amplitude Modulation is the process of changing the amplitude of a relatively high frequency carrier signal in proportion with the instantaneous value of the modulating signal.

- Define Modulation index and percent modulation for an AM wave. Mod-

Modulation index is a term used to describe the amount of amplitude change present in an AM waveform .It is also called as coefficient of modulation. Mathematically modulation index is

m =	Em Ec

Where m = Modulation coefficient

Em = Peak change in the amplitude of the output waveform voltage. Ec = Peak amplitude of the unmodulated carrier voltage.

Percent modulation gives the percentage change in the amplitude of the output wave when the carrier is acted on by a modulating signal.

- Define Low level Modulation.

In low level modulation, modulation takes place prior to the output element of the final stage of the transmitter. For low level AM modulator class A amplifier is used.

- Distinguish between low level and high level modulation.

In low level modulation, modulation takes place prior to the output element of the final stage of the transmitter. It requires less power to achieve a high percentage of modulation.

In high level modulators, the modulation takes place in the final element of the final stage where the carrier signal is at its maximum amplitude and thus

,requires a much higher amplitude modulating signal to achieve a reasonable percent modulation.

- Define Heterodyning.

Heterodyne means to mix two frequencies together in a nonlinear device or to translate one frequency to another using nonlinear mixing.

- Define direct frequency modulation.

In direct frequency modulation , frequency of a constant amplitude carrier signal is directly proportional to the amplitude of the modulating signal at a rate equal to the frequency of the modulating signal.

- Define indirect frequency Modulation.

In indirect frequency modulation ,phase of a constant amplitude carrier directly proportional to the amplitude of the modulating signal at a rate equal to the frequency of the modulating signal.

- What is multiplexing?

Multiplexing is the transmission of information from one or more source to one or more destination over the same transmission mediam.

- Define Open wire transmission line.

An open wire transmission line is a two wire parallel conductor. It consists simply of two parallel wires, closely spaced and separated by air. Nonconductive spacers are placed at periodic intervals for support and to keep the dielectric between the conductors constant. The dielectric is simply the air between and around the two conductors in which the TEM wave propagates.

- What are the advantages of open wire transmission line?

  - Simple in construction

  - Radiation losses are high

  - It is susceptible to noise pickups.

- Define propagation constant.

Propagation constant is used to express the attenuation (signal loss) and the phase shift per unit length of a transmission line. It is also called as propagation coefficient.

- Describe an incident wave, reflected wave.

An ordinary transmission line is bidirectional; power can propagate equally well in both directions. Voltage that propagates from the source toward the load is called incident voltage, and the voltage that propagates from the load toward the source is called reflected voltage.

- Define resonant line.

A transmission line with no reflected power is called a flat or resonant

line.

- Define matched line.

When Zo=ZL, all the incident power is absorbed by the load .This is called a matched line.

Where Zo=characteristic impedance ZL= load impedance

- Define unmatched line.

When Zo ≠ ZL, some of the incident power is absorbed by the load and some is returned to the source. This is called an unmatched or mismatched line.

Where Zo=characteristic impedance ZL= load impedance

- What are the advantages of digital transmission?

The advantage of digital transmission over analog transmission is noise immunity. Digital pulses are less susceptible than analog signals to variations caused by noise.

- Digital signals are better suitedto processing and multiplexing than analog signals.

- Digital transmission systems are more noise resistant than the analog transmission systems.

- Digital systems are better suited to evaluate error performance.

- Define pulse code modulation.

In pulse code modulation, analog signal is sampled and converted to fixed length, serial binary number for transmission. The binary number varies according to the amplitude of the analog signal.

- What is the purpose of the sample and hold circuit?

The sample and hold circuit periodically samples the analog input signal and converts those samples to a multilevel PAM signal.

- Define adaptive delta modulation.

Adaptive delta modulation is a delta modulation system where the step size of the AC is automatically varied depending on the amplitude characteristics of the analog input signal.

- Define Geo synchronous satellite.

Geo synchronous or geo stationary satellites are those that orbit in a

circular pattern with an angular velocity equal to that of Erath. Geosynchronous satellites have an orbital time of approximately 24 hours, the same as earth; thus geosynchronous satellites appear to be stationary as they remain in a fixed position in respect to a given point on earth.

- What are the advantages of optical fiber communication?

- Greater information capacity

- Immunity to crosstalk

- Immunity to static interference

- Environmental immunity

- Safety

- Security

PART C COMMUNICATION SYSTEMS

- Explain about Digital modulation techniques in detail?

- Explain about Analog modulation techniques in detail?

- Explain with neat diagram about Radio Transmitter?

- Explain with neat diagram about Radio Receiver?

- Explain with neat diagram about TV Transmitter?

- Explain with neat diagram about TV Receiver?

- Explain with neat diagram about satellite communication?