# UNIT 3-Diodes and Transistors

- Define Doping.

The process of adding other material to the crystal of Intrinsic Semiconductor to improve its conductivity is called doping. Impurity is called “dopant”.

# What is meant by semiconductor diodes?

The PN junction diode is one of the semiconductor devices with two semiconductor, materials in physical contact, one with excess of holes (P-type) and other with excess of electron (n-type).

# What is meant by Intrinsic semiconductor?

A pure semiconductor is called as intrinsic semiconductor. Even at the room temperature, some of the valence electrons may acquire   sufficient energy to enter the conduction band to form free electrons. A missing electron in the valence band leaves a vacant space which is known as hole. Holes also contribute to electric current.

# What is meant by Extrinsic semiconductor and mention its type.

Due to the poor conduction at room temperature, the intrinsic semiconductor as such, is not useful in the electronic devices. Hence the current conduction capability of the intrinsic semiconductor should be increased. This can be achieved by adding a small amount of impurity to the intrinsic semiconductor, so that it becomes impure or extrinsic semiconductor. The process of adding impurity is known as doping.

The types are

  - N-type semiconductor

  - P-type semiconductor.

# What is meant by PN Junction diode?

In a piece of semiconductor material if one half is doped by P-type impurity and other half is doped by N-type impurity, a PN Junction diode is formed. The plane dividing the two halves (or) zones is called PN Junction.

# What is meant by diffusion?

The N-type material has high concentration of free electron, while P-type material has. high concentration of holes. Therefore at the junction there is a tendency for the free electron to diffuse over the p-side and hole to the N-side. this process is called as diffusion

# What is meant by Reverse Saturation Current?

In a Reverse biased PN junction, a small amount of current flows due to the diffusion of minority charge carriers. The current produced is known as Reverse Saturation Current. The word saturation implies the reverse current cannot be increased by increasing reverse bias across the junction. But it can be increased with increase in temperature.

# Define Breakdown voltage.

When PN junction is reverse biased,a very small amount current called Reverse saturation current flows. This current is independent of applied voltage .However, if the reverse bias is increased to a large extent, break down occurs at junction, and the current through the junction increases abruptly. The voltage at which the breakdown occurs is known as “Breakdown voltage” and the breakdown is known as “Avalanche Breakdown”.

# Explain Avalanche Breakdown.

The reverse current is not dependant on reverse voltage. It depends on temperature. However if the reverse voltage is increased to a certain level, velocity of minority carriers increases. So minority carriers will move with high kinetic energy. When this carriers collide with atoms(SiorGe )with in the crystal, some more carriers (Electron –Hole pair) are created due to the breakage of bonds. These additional carriers pick up the energy from the applied voltage and generate still more carriers. This cumulative process of carrier generation or carrier multiplication is known as avalanche breakdown or avalanche multiplication .As a result of this reverse current increases rapidly.

# Define Barrier potential and Depletion Region.

As soon as PN junction is formed, some of the holes in the P region diffuse across the junction leaving –ve immobile ions in the P region..Similarly some of the electrons in the N region diffuse across the junction and reaches the P region leaving the +ve immobile ions in the Nside. In this process –ve acceptor ions in the P region and the +vedonar ions in the N region appears in the vicinity of the junction. The opposite charges existing near the junction creates a potential difference across the junction. This potential has a fixed polarity and it acts as a barrier to the flow of electrons and holes across the junction. Hence this is called barrier Potential (or) Junction Potential.

The region containing acceptor and donor ions in the vicinity of the junction is called “Depletion Region” or “Space charge Region”. Depletion region does not contain any charge carriers. It is fully depleted of charge carriers.

# Definecutinvoltage(or)Kneevoltage(or)thresholdoltage(or)Builtin voltage(or)Break overvoltage.

It is the voltage below which forward current remains low and if the applied forward bias voltage exceeds this value, forward current increases linearly with increases in forward voltage.

# Explain zener breakdown.

This type of break down occurs in junctions which are heavily doped. The heavily doped junction has narrow depletion layer. When the reverse voltage is increased, the electric field in the junction also increases. Electric field is voltage per distance and due to narrow depletion region and high reverse voltage it is intense(high). This electric field exerts a force on the electrons in the outermost shell. This force is so high, so that the electrons are pulled away from their parental atom and become free electrons. Such a creation of free electrons is called zener breakdown.

# What is zener diode?

Zener diode is a heavily doped silicon PN junction semiconductor device, which generally operates in its reverse breakdown region. The zener diodes are fabricated with precise breakdown voltages by controlling the doping level during manufacturing. In 1934, a physicist Carl Zener investigated the breakdown phenomenon in the PN junction diode.

# Compare zener diode and PN junction diode.

| ZENER DIODE | PN JUNCTION DIODE |
| --- | --- |
| - Operated in reverse breakdown region. - Dynamic zener resistance is very small in reverse breakdown condition. | - Operated in forward biased condition and never operated in reverse breakdown condition. - Diode resistance in reverse biased condition is very high. |
| - Conduction in zener is opposite to that of arrow in the symbol, as operated in breakdown region. | - Conduction when forward biased is in same direction as that of arrow in the symbol when forward biased. |
| - Used as voltage regulator, protection circuits and voltage limiters. | - Used as a rectifier , voltage multiplier , clipper and clamper. |

- What is a transistor?

It is a 3 layer, 3 terminal, and 2 junction device

.

# Why is the transistor called as the Bipolar Junction Transistor?(or) Why it is called as bipolar device?

The operation depends on the movement of both the majority and minority charge carriers.

# What are the three types of configuration in transistors?

Depending on the input, output and .common terminal a transistor are connected in 3 configurations;

- Common base configuration

- Common emitter configuration

- Common collector-Configuration.

# 18..What is FET and what type of device is this.

The device current is controlled by an electric field. So that this type of transistor is called as field effect transistor.

FET is a voltage controlled device because the output drain current can be controlled by input Gate Source Voltage.

# 19..Mention the application Applications of FET (JFET and MOSFET).

  - Used in chopper, sample and hold circuits.

  - Used in phase shift oscillators

  - Used as a voltage variable resistor in a FET voltmeter and AGC(Automatic Gain Control) circuits..

# 20..What is meant by MOSFET?

MOSFET – Metal oxide semiconductor FET

A thin layer of metal oxide is deposited over the channel. It is usually silicon dioxide. A metallic gate is mounted over the metal oxide layer which acts as the insulator. Therefore gate is insulated from the channel. Hence MOSFET is also called as Insulated Gate FET (IGFET).

# 21..Compare FET and BJT.

| FET | BJT |
| --- | --- |
| - Unipolar device i.e. current in the device carried by electrons or holes - p-channel->holes - n-channel->electrons | 1. Bipolar device i.e. current in the device is carried by both the electrons and the holes. |
| 2. It is the voltage controlled device i.e. voltage at the gate (or drain) terminal controls the amount of current flowing through the device. | 2. It is a current controlled device i.e. the base current controls the amount of base current. |
| 3. Input resistance is very high order of several mega ohms | 3. Very low input as compared to FET order of few kilo ohms. |
| 4. It has –ve temperature co efficient at high currents level. i.e. current decreases as the temperature increases, so no thermal breakdown or secondary breakdown | 4. It has +ve temperature co efficient at a high current level. That means collector current increases with increase in temperature. This characteristics leads to secondary breakdown in BJT |
| 5. Higher switching speed | 5. lower switching speed |
| 6. Less noisy | 6. Comparatively more noisy. |

PART C

- Explain the operation of NPN transistor in CB configuration with its input and output characteristics.

- Explain the working of PN junction diode and its VI characteristics

- With a neat sketch explain the construction and characteristics of MOSFET.

- Explain the construction and working of Zener diode with a neat sketch

- Explain the input and output characteristics of a CE transistor configuration. List out the comparisons between CE, CB and CC configurations

- Explain the construction of N channel JFET and also explain the drain and transfer characteristics of the same

- Describe the static input and output characteristics of a CB transistor with neat circuit diagram.

8.(i)Distinguish between avalanche and zener mechanisms. (ii)Explain how barrier potential is developed at the PN junction.